# HESSI Spectrometer Integration Procedure

HSI-INT-PROC-001

Revision: v1    Date: 2025-03-01

Prepared by: Spectrum Astro Integration Team

# Table of Contents

1. Scope

2. Prerequisites

3. Mechanical Installation

4. Electrical Connections

5. Alignment Verification

# Scope

This procedure defines the step-by-step process for integrating the Spectrometer assembly onto the HESSI spacecraft bus. It covers mechanical installation, electrical connections, alignment verification, and functional checkout.

# Prerequisites

- Spacecraft bus in integration configuration

- Spectrometer assembly passed standalone functional test

- Clean room environment (Class 10,000 or better)

- Alignment GSE available and calibrated

- Shimming materials available (0.001 to 0.020 inch range)

# Mechanical Installation

Step 1: Position the Spectrometer above the spacecraft interface using the handling fixture.

Step 2: Lower the Spectrometer onto the four mounting pad interface points.

Step 3: Install the 8 mounting bolts finger-tight.

Step 4: Perform preliminary alignment check using theodolite.

Step 5: Install shims as required to achieve 1mm concentricity.

Step 6: Torque all mounting bolts to 45 in-lbs.

# Electrical Connections

Step 7: Connect the power harness (J1 connector) to the CPC interface.

Step 8: Connect the data harness (J2 connector) to the IDPU interface.

Step 9: Connect the heater harness (J3 connector) to the thermal control bus.

Step 10: Verify all connections with continuity check.

# Alignment Verification

Step 11: Perform final alignment survey using laser tracker.

Step 12: Verify detector array concentricity is within 1mm of grid pair centers.

Step 13: Record alignment data in traveler document.