# HESSI Spectrometer Mechanical Interface Requirements

HSI-MECH-REQ-001

Revision: v3    Date: 2024-11-05

Prepared by: UCB Mechanical Engineering

# Table of Contents

1. Scope

2. Interface Drawing

3. Mass Properties

4. Field of View and Alignment

5. Mechanisms

# Scope

This document defines the mechanical interface requirements between the HESSI Spacecraft Bus and the Spectrometer instrument assembly. It establishes the physical constraints, mounting interface, mass allocation, and alignment requirements.

Change from v2: All TBRs resolved. Final values from qualification testing incorporated. Document approved for ICD baseline.

# Interface Drawing

The mechanical configuration of the Spectrometer is shown in the Spectrometer ICD Drawing (HSI_SYS_014F, Rev 3). Drawing updated 2024-09-15 incorporating CDR redlines.

# Mass Properties

The Spectrometer assembly mass allocation is 50 kg maximum.

Final Measured Mass: 45.2 kg (flight unit weigh)

Margin: 9.6%

Component breakdown (measured):

- Cryostat Assembly: 29.1 kg

- Detector Array (9x): 4.3 kg

- Cryocooler: 6.9 kg

- Hi-Z Shutters: 1.08 kg

- Thermal Hardware: 2.0 kg

- Harnesses: 1.82 kg

Note: Mass margin below 10% accepted per waiver MSE-2024-003.

# Field of View and Alignment

The nine detectors must be aligned to be within 1mm of concentric to the field of view of each of the nine grid pairs on the imager. This alignment is achieved when installing the Spectrometer on the spacecraft by shimming at the imager interface points.

Verified during instrument-level alignment test (ref: HSI-ALN-RPT-001). Achieved alignment: 0.4mm RMS.

# Mechanisms

The Spectrometer contains the following mechanisms:

- Cryocooler: free piston, linear motion, He gas compressor. Residual operating forces: 0.48 newtons at 59 Hz (measured during qualification vibration test, satisfies former TBR-UCB-102 requirement of 0.5N max).

- Hi-Z Shutters (2x): moving mass 300g (thick) and 230g (thin). Measured actuation time: 0.47 seconds (thick), 0.44 seconds (thin).

- Cryostat Vacuum Valve: SMA-actuated one-shot valve. Activation delay: 96 hours minimum post-launch (thermal settling requirement).