# HESSI Spectrometer Mechanical Interface Requirements

HSI-MECH-REQ-001

Revision: v4    Date: 2025-06-15

Prepared by: UCB Mechanical Engineering

# Table of Contents

1. Scope

2. Interface Drawing

3. Mass Properties

4. Field of View and Alignment

5. Mechanisms — On-Orbit Performance

# Scope

This document defines the mechanical interface requirements between the HESSI Spacecraft Bus and the Spectrometer instrument assembly.

Change from v3: Updated with as-flown data from first 6 months of operations. All requirements verified on-orbit.

# Interface Drawing

The mechanical configuration of the Spectrometer is shown in the Spectrometer ICD Drawing (HSI_SYS_014F, Rev 3). No changes from v3 — drawing represents as-built configuration.

# Mass Properties

The Spectrometer assembly mass allocation is 50 kg maximum.

Final Measured Mass: 45.2 kg (unchanged from v3)

On-orbit mass verification: confirmed via spacecraft momentum management data.

All mass values represent as-flown configuration.

# Field of View and Alignment

The nine detectors must be aligned to be within 1mm of concentric to the field of view of each of the nine grid pairs on the imager.

On-orbit verification: Alignment verified via star tracker cross-calibration during commissioning. Measured on-orbit alignment: 0.6mm RMS (within 1mm requirement). Slight degradation from ground measurement (0.4mm) attributed to launch shift — within predictions.

# Mechanisms — On-Orbit Performance

Cryocooler: Operating nominally at 59 Hz. Measured residual force: 0.42N (improved from ground test due to thermal settling). Zero vibration complaints from spacecraft bus.

Hi-Z Shutters: Both shutters actuated successfully during commissioning. Thick shutter: 0.48s actuation. Thin shutter: 0.45s. Both within spec.

Cryostat Vacuum Valve: Opened successfully at L+120 hours. Cryostat pressure: <1e-6 torr within 24 hours of valve opening.